Name                 : Jaswanth babu R R

Reg No              :192512341

Course name     : Advanced Wireless Communincation system

Course code      : ECA5610

MATLAB Analysis of Spectrum Bands, Propagation Loss and Cellular Interference

Aim:

# To compare a simplified free-space path-loss estimate across representative sub-1 GHz, mid-band and mmWave frequencies, and examine how distance and frequency reuse affect received signal and a simple interference measure.

# Objectives:

1. Calculate or simulate path loss at representative sub-1 GHz, mid-band and mmWave frequencies.

2.  Compare estimated received signal strength as frequency and distance change.

3.  Examine how cell separation or frequency reuse changes a simplified interference ratio.

4.  Relate the results to coverage and spectrum-use considerations across mobile generations, while stating model limitations

SOFTWARE USED:

  - Computer with MATLAB

# Program :

clear; clc; close all;

% Representative bands and link-budget assumptions

freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"];

txPowerdBm = 30;

gtDbi = 0;

grDbi = 0;

distanceKm = logspace(log10(0.1), log10(2), 200);

distanceAtPointKm = 0.5;

% Free-space path loss (d in km, f in MHz)

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

lossAtPoint = fspl(freqMHz, distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

linkTable = table(bandName, freqGHz(:), lossAtPoint(:), rxAtPoint(:), ...

'VariableNames', {'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'});

disp(linkTable);

% Path loss and received power over distance

figure;

tiledlayout(1,2);

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

plot(distanceKm, fspl(freqMHz(k), distanceKm), 'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Path loss (dB)');

legend(bandName, 'Location', 'northwest');

title('Free-Space Path Loss');

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

rx = txPowerdBm + gtDbi +

# Output:

+